SMARTHIKA AGRO

MQTT Payload Reference

Schema v3.0  •  Gateway Controller GRC_SMARTHIKA_SPI_VI_V3  •  April 2026

| Field | Value |
| --- | --- |
| Broker | AWS IoT Core (ap-south-1) |
| Protocol | MQTT 3.1.1 over TLS / mTLS |
| Schema Version | v3.0 — all payloads carry "v": "3.0" |
| Device | GRC_SMARTHIKA_SPI_VI_V3 |
| Client ID format | gw-motor-ctrl-{6-digit-serial} e.g. gw-motor-ctrl-373050 |
| Timestamp format | ISO 8601 UTC compact e.g. 20260413T160000Z |
| Changes from v2.1 | New topics: /display, /ota, /yieldtest, /recovery. Health lean schema. E-codes on error. User context on cmd. boot.* fields on health. Battery/RSSI moved to telemetry. |

1. MQTT Topic Map

All topics use a hierarchical namespace under gw/{gw_id}/. Replace {gw_id} with the device client ID. {node_id} and {valve_id} are used for LoRa peripheral topics.

## 1.1  Device → Cloud (Published by Firmware)

| Topic | QoS | Cadence | Description |
| --- | --- | --- | --- |
| gw/{gw_id}/telemetry | 1 | 5s run / 10s idle | Primary heartbeat. Electrical, hydraulic, motor, network state. Battery/RSSI every 60s (cadence-gated optional fields). |
| gw/{gw_id}/status | 1 | On event | Motor state change, mode change, or active_test change. Published immediately. Authoritative source for motor and mode state. |
| gw/{gw_id}/health | 0 | Boot + every 1hr | Lean diagnostics: boot hardware flags, firmware version, SIM ICCID, IP address, GSM RSSI. Uptime. |
| gw/{gw_id}/error | 1 | On fault only | Fault alert with E-code, string name, category A/B/C/D, severity, and electrical snapshot at fault time. |
| gw/{gw_id}/response | 1 | After every cmd | ACK or NACK for each received command. Echoes request_id and cmd. Carries reason_code on rejection. |
| gw/{gw_id}/ota | 1 | On OTA event | Firmware upgrade state: started / success / failed. Published before beginning flash and after completion. |
| gw/{gw_id}/yieldtest | 0 | Every 60s during test | Step drawdown data streamed during yield test steps. Source for Jacob well loss and Specific Capacity calculation. |
| gw/{gw_id}/recovery | 0 | Every 60s post-pump | Water level recovery curve after every pump-off. Source for Theis transmissivity and recharge rate computation. |
| gw/{gw_id}/networktest | 0 | Every 60s during test | Observation bore level change during inter-bore transmissivity network test. |

## 1.2  Gateway → Cloud (Forwarded by Firmware from Peripherals)

| Topic | QoS | Cadence | Description |
| --- | --- | --- | --- |
| nodes/{node_id}/env | 0 | On LoRa RX | Environmental data from LoRa soil/weather nodes: soil moisture, temperature, rain intensity, solar lux, node battery. |
| valves/{valve_id}/status | 1 | On valve change | Valve open/close state from LoRa or RS485 actuators. Includes ACK string and RSSI for link quality. |

## 1.3  Cloud → Device (Published by Lambda or App via Broker)

| Topic | QoS | Cadence | Description |
| --- | --- | --- | --- |
| gw/{gw_id}/cmd | 1 | On command | All device commands. Lambda validates user role before forwarding. Carries farm_id, user_id, request_id for audit. |
| gw/{gw_id}/display | 1 | On state change | Cloud Lambda origin only — firmware never publishes here. LED array + optional LCD text override for the app. |

2. Measured Parameters — Hardware Sources

All measured values come from hardware peripherals. The table below maps each parameter to its JSON key in the telemetry payload, unit, hardware source function, and production status.

## 2.1  Electrical — ATM90E36A Energy Meter (SPI)

| Parameter | JSON Key | Unit | Getter Function | Status |
| --- | --- | --- | --- | --- |
| Phase R Voltage (RMS) | el.v[0] | V | ATM90E36_GetLineVoltageA() | READY |
| Phase Y Voltage (RMS) | el.v[1] | V | ATM90E36_GetLineVoltageB() | READY |
| Phase B Voltage (RMS) | el.v[2] | V | ATM90E36_GetLineVoltageC() | READY |
| Phase R Current (RMS) | el.i[0] | A | ATM90E36_GetLineCurrentA() | READY |
| Phase Y Current (RMS) | el.i[1] | A | ATM90E36_GetLineCurrentB() | READY |
| Phase B Current (RMS) | el.i[2] | A | ATM90E36_GetLineCurrentC() | READY |
| Grid Frequency | el.f | Hz | ATM90E36_GetFrequency() | READY |
| Phase Sequence | el.seq | — | ATM90E36_GetPhaseSequence() | READY |
| System Status Register 0 | el.sys0 | hex | ATM90E36_GetSysStatus0() | READY |
| Power Factor (derived) | el.pf | 0–1.0 | cos(PhaseAngle × π/180) avg of 3 phases | FIRMWARE ADD |
| Total Active Power (derived) | el.tp | W | V[n] × I[n] × PF[n] summed | FIRMWARE ADD |
| Current Imbalance (derived) | el.imb | % | (max(I)-min(I))/avg(I)×100 | FIRMWARE ADD |

## 2.2  Hydraulic — RS485 Pressure Sensor

| Parameter | JSON Key | Unit | Source | Status |
| --- | --- | --- | --- | --- |
| Discharge Pressure (raw ADC) | hy.p | Bar | ParsedResponse_t.raw_pressure_adc converted | READY |
| Static Head | hy.st_h | m | Last hy.p reading at motor idle × 10.2 m/Bar | FIRMWARE ADD |
| Flow Rate | hy.fl_m | m³/hr | Pulse-counter GPIO — reed switch flow meter | NEW HW NEEDED |
| Flow Totalizer | hy.vol_t | Litres | Accumulated from flow pulses stored in NVRAM | NEW HW NEEDED |

## 2.3  Motor Manager

| Parameter | JSON Key | Unit | Getter Function | Status |
| --- | --- | --- | --- | --- |
| Motor State | m.st | string | Motor_GetState() → mapped string | READY |
| Cumulative Runtime | m.rt | sec | Motor_GetTotalRuntime() / 1000 | READY |
| Session Runtime | m.srt | sec | Motor_GetStateTime() / 1000 | READY |
| Operation Mode | m.op | string | Motor_GetOperationModeString() | READY |
| Dry Run Count | m.dry | int | Static counter — increments on dry run state entry | FIRMWARE ADD |

## 2.4  Power Manager

| Parameter | JSON Key | Unit | Getter | Status |
| --- | --- | --- | --- | --- |
| Backup Battery Voltage | sys.bv | V | PowerManager_GetBatteryVoltage() | READY — 60s gated in telemetry |
| Battery Percentage | sys.bp | % | PowerManager_GetBatteryPercentage() | READY — 60s gated in telemetry |
| System Uptime | sys.upt | sec | HAL_GetTick() / 1000 | READY |

## 2.5  GSM / Network

| Parameter | JSON Key | Unit | Source | Status |
| --- | --- | --- | --- | --- |
| GSM Connection State | net.connected | bool | GSM_GetState() = IDLE_CONNECTED | READY |
| GSM Signal Strength | sys.net_s | dBm | AT+CSQ parsed — GSM_GetRSSI() getter | FIRMWARE ADD — 60s gated |
| IP Address | net.ip (health) | string | AT+QIACT parsed and stored | FIRMWARE ADD |
| SIM ICCID | net.sim (health) | string | AT+QCCID parsed at boot | FIRMWARE ADD |

3. Full Payload Schemas — v3.0

## 3.1  Telemetry — gw/{gw_id}/telemetry

Trigger: Every 5s when motor running. Every 10s when idle. sys.bv, sys.bp, sys.net_s every 60s only (cadence-gated).

// Topic: gw/gw-motor-ctrl-373050/telemetry
{
  "v": "3.0",
  "ts": "20260413T160000Z",          // ISO timestamp from RTC
  "el": {
    "v": [230.5, 231.2, 229.8],      // Phase R, Y, B voltages (V)
    "i": [4.21, 4.25, 4.22],         // Phase R, Y, B currents (A)
    "f": 50.02,                       // Grid frequency (Hz)
    "pf": 0.88,                       // Power factor — cos(PhaseAngle avg)
    "tp": 2856.4,                     // Total active power (W)
    "imb": 0.9,                       // Current imbalance (%)
    "seq": "ABC"                      // Phase sequence: "ABC" or "ACB"
  },
  "hy": {
    "p": 4.5,                         // Discharge pressure (Bar)
    "st_h": 21.34                     // Static head (m) — water column above sensor at idle
  },
  "m": {
    "st": "run_d",                    // idle | run_s | run_d | fault
    "rt": 86400,                      // Cumulative runtime (sec)
    "srt": 3600,                      // Session runtime (sec) — resets on each motor start
    "op": "star_delta",               // star_delta | single_contactor
    "dry": 2                          // Dry run event count (cumulative)
  },
  "net": {
    "connected": true                 // GSM connection state
  },
  "sys": {                            // sys.bv, sys.bp, sys.net_s: every 60s only
    "upt": 172800,                    // Uptime since boot (sec) — every publish
    "bv": 7.4,                        // Battery voltage (V) — 60s gate
    "bp": 85,                         // Battery percentage (%) — 60s gate
    "net_s": -67                      // GSM RSSI (dBm) — 60s gate
  }
}

## 3.2  Status — gw/{gw_id}/status

Trigger: On motor state change, mode change, or active_test change. Authoritative for LED mode/motor decisions.

// Topic: gw/gw-motor-ctrl-373050/status
{
  "v": "3.0",
  "ts": "20260413T160000Z",
  "motor": {
    "st": "run_d",                    // idle | run_s | run_d | fault
    "active": true                    // false when idle or fault
  },
  "mode": "auto",                     // auto | manual
  "active_test": null,                // null | "yield_test" | "recovery"
  "trigger": "motor_start"            // What caused this publish
}

active_test distinguishes a yield test motor run from a normal motor run. The LED sequences are identical — active_test is the only signal the app can use to switch buttons to Abort Test mode.

## 3.3  Health — gw/{gw_id}/health  (Lean v3.0)

Trigger: Once on boot, then every 1 hour. boot object is populated on boot publish only — null on hourly.

// Topic: gw/gw-motor-ctrl-373050/health
{
  "v": "3.0",
  "ts": "20260413T040000Z",
  "boot": {                           // null on hourly publish
    "gsm_hw": true,                   // GSM hardware detected on UART
    "lora_hw": true,                  // LoRa module detected
    "pri_ok": true,                   // Primary network (GSM) init OK
    "sec_ok": false                   // Secondary network (LoRa) init status
  },
  "net": {
    "s": -67,                         // GSM RSSI (dBm)
    "ip": "10.45.22.1",              // Assigned IP address
    "sim": "8991450300..."            // SIM ICCID — for AWS IoT auth
  },
  "sys": {
    "fw": "1.2.0",                    // Firmware version — used for OTA check
    "upt": 172800                     // Uptime since boot (sec)
  }
  // REMOVED from v2.1: sys.ac, sys.bv, sys.bp, conf.mode, net.t
  // sys.bv + sys.bp + net_s moved to telemetry sys block (60s gated)
}

## 3.4  Error — gw/{gw_id}/error

Trigger: On fault detection. Always QoS 1. e_code is canonical identifier. cat carries fault category A/B/C/D.

// Topic: gw/gw-motor-ctrl-373050/error
{
  "v": "3.0",
  "ts": "20260413T160000Z",
  "e_code": "E0001",                 // Canonical E-format fault code
  "code": "OVERVOLTAGE_A",           // String name for developer reference
  "sev": "critical",                 // critical | warning | info
  "cat": "A",                        // A=HardStop B=SoftStop C=Warning D=Connectivity
  "mode": "auto",                    // Device mode when fault occurred
  "desc": "Phase A voltage exceeded 260V",
  "snap": {                          // Electrical state at exact fault moment
    "v": [262.1, 230.2, 231.0],      // Voltages at fault — drives phase LED state
    "i": [4.10, 4.05, 4.08],         // Currents at fault
    "f": 50.01                       // Frequency at fault
  }
}

Lambda uses snap.v[] to correctly set phase LEDs in the display payload — phase LEDs reflect the actual electrical state at fault time, not the current state.

## 3.5  Command — gw/{gw_id}/cmd

Direction: Cloud/App → Device. Lambda validates farm_id + user_id role before forwarding. Firmware ignores farm_id and user_id.

// Topic: gw/gw-motor-ctrl-373050/cmd
{
  "v": "3.0",
  "cmd": "PUMP_START",               // See command list in Section 4
  "target": "gw-motor-ctrl-373050",
  "farm_id": "farm-krishna-001",     // Lambda role check — not used by firmware
  "user_id": "usr-john-001",          // Audit trail — not used by firmware
  "request_id": "REQ-uuid-a1b2c3",  // UUID per tap — echoed in response for ACK matching
  "params": {},                      // Command-specific params (see Section 4)
  "ts": "20260413T160000Z"
}

## 3.6  Response — gw/{gw_id}/response

Trigger: After every received command. QoS 1. request_id echoed for ACK matching.

// Topic: gw/gw-motor-ctrl-373050/response
{
  "v": "3.0",
  "ts": "20260413T160005Z",          // Firmware processing timestamp
  "request_id": "REQ-uuid-a1b2c3",  // Echoed from cmd payload
  "cmd": "PUMP_START",               // Echoed command for context
  "result": "ok",                    // ok | rejected | error
  "message": "Motor started",
  "reason_code": null                // On rejected: see reason codes below
}

      
// reason_code values on rejected:
// ACTIVE_FAULT | PHASE_MISSING | SDL_TOO_LOW | INSUFFICIENT_ROLE |
// DEVICE_OFFLINE | YIELD_TEST_ACTIVE | OTA_IN_PROGRESS | RESTART_LOCKOUT

## 3.7  Display — gw/{gw_id}/display

Origin: Cloud Lambda only. Never published by firmware. lcd: null = app mirrors telemetry. lcd object = cloud overrides LCD panel.

// Normal operation — LEDs only, no LCD override:
{
  "v": "3.0",
  "ts": "20260413T160000Z",
  "leds": [1, 1, 1, 0, 0, 1, 0, 1, 1, 0],  // All 10 LEDs: 1=ON 0=OFF
  "lcd": null                               // null = app mirrors telemetry text
}

      
// Cloud LCD override (OTA / fault / yield test screens):
{
  "v": "3.0",
  "ts": "20260413T040001Z",
  "leds": [0, 0, 0, 0, 0, 0, 0, 0, 0, 0],
  "lcd": {
    "row0": "FW UPGRADE...",               // Max 16 chars — mirrors physical LCD
    "row1": "DO NOT POWER OFF"             // Max 16 chars
  }
}

      
// leds[] index mapping:
// [0]=LINE1(R) [1]=LINE2(Y) [2]=LINE3(B) [3]=MTR_ON [4]=TRIP
// [5]=SMART_MODE [6]=MANUAL_MODE [7]=NET1(GSM) [8]=NET2(LoRa) [9]=NET3

## 3.8  OTA — gw/{gw_id}/ota

Trigger: Firmware publishes before beginning flash, after success, and on failure.

// state: "started" — published before flash begins
{ "v":"3.0", "ts":"...", "state":"started", "fw_current":"1.1.0", "fw_target":"1.2.0" }

      
// state: "success" — published after flash, before reboot
{ "v":"3.0", "ts":"...", "state":"success", "fw_current":"1.1.0", "fw_target":"1.2.0" }

      
// state: "failed" — published before falling back to existing firmware
{ "v":"3.0", "ts":"...", "state":"failed",  "fw_current":"1.1.0", "fw_target":"1.2.0",
  "reason":"checksum_mismatch" }   // checksum_mismatch | download_fail | flash_fail

## 3.9  Yieldtest — gw/{gw_id}/yieldtest

Trigger: Every 60s during each yield test step. Active only during YIELD_TEST_START … YIELD_TEST_STOP window.

{ "v": "3.0",
  "ts": "20260413T061000Z",
  "step": 1,                         // Step number 1–4
  "flow_m3hr": 2.0,                  // Flow rate at this step (set by NEXTSTEP cmd)
  "water_col_m": 19.2,               // hy.p × 10.2 — live DDL water column above sensor
  "drawdown_m": 2.14,                // sdl_col_m - water_col_m
  "elapsed_sec": 600,                // Time since this step started
  "sdl_col_m": 21.34                 // SDL baseline captured at test start from hy.st_h
}

## 3.10  Recovery — gw/{gw_id}/recovery

Trigger: Every 60s after every pump-off for up to 90 min. Published after normal stops AND yield tests.

{ "v": "3.0",
  "ts": "20260413T110000Z",
  "water_col_m": 17.5,               // Current water column — rising as bore recovers
  "elapsed_sec": 600,                // Time since pump stopped
  "trigger": "post_yield",           // post_pump | post_yield | post_test
  "pump_off_col_m": 15.2             // Water level at exact pump-off moment — Theis t=0
}

## 3.11  Environmental Node — nodes/{node_id}/env

Trigger: On LoRa RX. Data originates from remote LoRa soil/weather sensors and is forwarded by the gateway.

{ "v": "3.0", "node_id": "snr-soil-01", "ts": "...",
  "soil": { "d10": 28.5, "d30": 24.2, "temp": 22.5 },  // Moisture % at depth, soil temp C
  "atm": { "t": 32.5, "h": 45, "r": 0.0, "lx": 15000 },// Ambient temp,humidity,rain,lux
  "bat": 3.8, "rssi": -95             // Node battery V and LoRa RSSI
}

## 3.12  Valve Status — valves/{valve_id}/status

{ "v": "3.0", "v_id": "valve-zone-03", "ts": "...",
  "st": 1,                            // 0=Closed 1=Open 2=Fault
  "src": "lora",                      // lora | rs485 | wifi
  "bat": 3.8, "rssi": -105, "ack": "08:1"
}

4. Command Reference — gw/{gw_id}/cmd

All commands share the base structure in Section 3.5. The params object varies per command. Commands are role-gated by Lambda before reaching the device. The device echoes request_id in the response for ACK matching.

| cmd | params fields | Allowed Roles | Firmware Action |
| --- | --- | --- | --- |
| PUMP_START | No params required | owner, operator, technician | Validates phase + fault locally. Energises star contactor. Publishes status. |
| PUMP_STOP | No params required | owner, operator, technician | De-energises relay immediately. Publishes status. Starts recovery log timer. |
| MODE_CHANGE | { "mode": "auto" ¦ "manual" } | owner, operator, technician | Sets mode flag. Locks/unlocks SET button on device. Publishes status. |
| CMD_FAULT_RESET | No params required | owner, operator, technician | Clears FaultManager latch. Re-arms protection. Publishes status motor.st=idle. |
| OTA_START | { "fw_version":"1.2.0", "url":"https://...", "checksum":"sha256:..." } | owner, technician | Validates checksum. Begins download. Publishes ota started. Telemetry pauses. |
| YIELD_TEST_START | { "step_count":4, "step_duration_sec":3600, "flow_steps_m3hr":[2.0,3.0,4.5,6.0] } | owner, technician | Starts motor. Enters yield test state. Publishes status active_test=yield_test. |
| YIELD_TEST_NEXTSTEP | { "step":2, "flow_m3hr":3.0 } | Cloud Lambda only | Adjusts flow setting. Resets step timer. Continues running. |
| YIELD_TEST_STOP | No params required | owner, operator, technician, Lambda | Stops pump. Clears yield test state. Publishes status active_test=null. |
| DUTY_CYCLE_UPDATE | { "on_time_sec":195, "off_time_sec":4500 } | Cloud Lambda only | Stores to NVM. Applies immediately to scheduler. |
| VALVE_CTRL | { "v_id":"valve-zone-03", "v_state":1, "dur":120 } | owner, operator, technician | Forwards valve command to LoRa/RS485 module. |
| SCHEDULE_SET | { "schedule":[{"start":"06:00","dur_min":30},...] } | owner, technician | Stores schedule to NVM. Triggers based on RTC. |

5. Fault E-Code Reference

E-codes are the canonical fault identifiers. Both the app and Lambda maintain a lookup table mapping E-code to description, LCD text, phase LED state, and banner colour. The device always sends both e_code and code (string name) in the error payload.

Category A = Hard Stop (relay trips always). Category B = Soft Stop (cloud sends PUMP_STOP, no latch). Category C = Warning (motor continues, alert only). Category D = Connectivity (no motor effect). In Manual mode, Category B and C faults generate alerts but do NOT auto-stop — relay decision belongs to the operator. Category A always trips regardless of mode.

## 5.1  Electrical Faults (E0001–E0018)

Source: ATM90E36A via SPI. Thresholds applied by FaultManager on el.v[], el.i[], el.f, el.seq.

| E-Code | String Name | Cat | LCD Row 1 | Phase LEDs | TRIP | Threshold / Condition |
| --- | --- | --- | --- | --- | --- | --- |
| E0001 | OVERVOLTAGE_A | A | Overvoltage R | LINE 1 ON | 1 | el.v[0] > 260V sustained 2s. Both modes: relay trips. |
| E0002 | OVERVOLTAGE_B | A | Overvoltage Y | LINE 2 ON | 1 | el.v[1] > 260V sustained 2s. |
| E0003 | OVERVOLTAGE_C | A | Overvoltage B | LINE 3 ON | 1 | el.v[2] > 260V sustained 2s. |
| E0004 | UNDERVOLTAGE_A | A | Undervoltage R | LINE 1 OFF | 1 | el.v[0] < 180V while running. Auto: trips. Manual: alert only. |
| E0005 | UNDERVOLTAGE_B | A | Undervoltage Y | LINE 2 OFF | 1 | el.v[1] < 180V while running. Auto: trips. Manual: alert. |
| E0006 | UNDERVOLTAGE_C | A | Undervoltage B | LINE 3 OFF | 1 | el.v[2] < 180V while running. Auto: trips. Manual: alert. |
| E0007 | PHASE_LOSS_A | A | Phase Loss R | LINE 1 OFF | 1 | el.v[0] < 50V. Hardware safety — trips both modes. |
| E0008 | PHASE_LOSS_B | A | Phase Loss Y | LINE 2 OFF | 1 | el.v[1] < 50V. Hardware safety — trips both modes. |
| E0009 | PHASE_LOSS_C | A | Phase Loss B | LINE 3 OFF | 1 | el.v[2] < 50V. Hardware safety — trips both modes. |
| E0010 | PHASE_SEQUENCE | A | Phase Seq Fault | All 3 ON | 1 | el.seq = ACB at startup. Motor start blocked both modes. |
| E0011 | OVERCURRENT_A | A | Overcurrent R | All 3 ON | 1 | el.i[0] > rated × 1.2 sustained 3s. Auto: trips. Manual: alert. |
| E0012 | OVERCURRENT_B | A | Overcurrent Y | All 3 ON | 1 | el.i[1] > rated × 1.2 sustained 3s. |
| E0013 | OVERCURRENT_C | A | Overcurrent B | All 3 ON | 1 | el.i[2] > rated × 1.2 sustained 3s. |
| E0014 | CURRENT_IMBALANCE | B | Current Imbalance | All 3 ON | 1 | el.imb > 15% for 10s during run. Auto: soft stop. Manual: alert only. |
| E0015 | UNDERCURRENT | B | Undercurrent | All 3 ON | 1 | el.i avg < 20% rated for 5s — pipe burst or open valve. Auto: soft stop. Manual: alert. |
| E0016 | LOW_FREQUENCY | C | Low Frequency | All 3 ON | 0 | el.f < 48Hz for 5s. Warning only — motor continues. Both modes. |
| E0017 | HIGH_FREQUENCY | C | High Frequency | All 3 ON | 0 | el.f > 52Hz for 5s. Warning only. |
| E0018 | VOLTAGE_IMBALANCE | C | Voltage Imbalance | Dim affected | 0 | (max(V)-min(V))/avg(V) > 5%. Warning only. Both modes monitor. |

## 5.2  Motor Operation Faults (E0101–E0109)

Source: Motor Manager state machine transitions.

| E-Code | String Name | Cat | LCD Row 1 | Phase LEDs | TRIP | Condition |
| --- | --- | --- | --- | --- | --- | --- |
| E0101 | ALARM_DRY_RUN | A | Dry Run | All 3 ON | 1 | Current below dry-run threshold (~30% rated) while running. Both modes: relay trips — bore damage prevention. |
| E0102 | START_FAIL | A | Start Failed | All 3 ON | 1 | Motor did not reach run_d within star-delta timeout (~10s). Both modes. |
| E0103 | STAR_DELTA_FAIL | A | SD Transition Fail | All 3 ON | 1 | Transition from run_s to run_d timed out — contactor or wiring fault. |
| E0104 | OVERLOAD | A | Motor Overload | All 3 ON | 1 | Current > 150% rated for > 10s. Auto: trips. Manual: alert + 10s operator window. |
| E0105 | LOCKED_ROTOR | A | Rotor Locked | All 3 ON | 1 | Current > 200% rated in first 3s of start. Both modes: trips immediately. |
| E0106 | THERMAL_OVERLOAD | A | Thermal Overload | All 3 ON | 1 | I²t accumulator exceeds winding thermal limit. Auto: trips. Manual: alert + 5s grace. |
| E0107 | RESTART_LOCKOUT | B | Restart Lockout | All 3 ON | 0 | > 3 start attempts in 10 min — cooling lockout. Auto: blocks start. Manual: warning, one override allowed. |
| E0108 | DDL_LIMIT | B | DDL Limit Reached | All 3 ON | 0 | hy.p × 10.2 <= min_safe_col_m. Cloud-detected. Auto: cloud sends PUMP_STOP. Manual: alert only. |
| E0109 | DRAWDOWN_ACCEL | B | Drawdown Alert | All 3 ON | 0 | Drawdown rate > 2x baseline over 5 readings. Cloud-detected. Auto: cloud sends PUMP_STOP. Manual: alert. |

## 5.3  Power Supply Faults (E0201–E0204)

Source: Power Manager state transitions.

| E-Code | String Name | Cat | LCD Row 1 | Phase LEDs | TRIP | Condition |
| --- | --- | --- | --- | --- | --- | --- |
| E0201 | MAINS_LOSS | A | Mains Lost | All 3 OFF | 1 | sys.ac = false — all phases gone. Both modes: relay trips — no supply. |
| E0202 | POWER_BROWN_OUT | A | Brown Out | All 3 dim | 1 | All phases present but all below 170V. Auto: trips immediately. Manual: alert + operator. |
| E0203 | BATTERY_LOW | C | Battery Low | Unchanged | 0 | sys.bp < 20%, sys.bv < 6.8V. Warning only. GSM still working. Both modes. |
| E0204 | BATTERY_CRITICAL | B | Battery Critical | Unchanged | 0 | sys.bp < 5%, sys.bv < 6.2V. Device may lose GSM soon. Cloud notifies all roles. |

## 5.4  Hydraulic Faults (E0301–E0303)

Source: RS485 Pressure Sensor.

| E-Code | String Name | Cat | LCD Row 1 | Phase LEDs | TRIP | Condition |
| --- | --- | --- | --- | --- | --- | --- |
| E0301 | PRESSURE_SENSOR_FAIL | B | Pressure Sensor | Unchanged | 0 | RS485 no response for 3 consecutive reads. hy.p = null. Cloud disables DDL protection. |
| E0302 | PRESSURE_OVERRANGE | A | Over Pressure | All 3 ON | 1 | hy.p > sensor max range — pipe blockage or valve closed. Both modes: trips. |
| E0303 | NO_FLOW_RUNNING | B | No Flow Detected | All 3 ON | 0 | Motor running but hy.p near zero — cavitation risk. Requires flow sensor HW. Auto: soft stop. |

## 5.5  Connectivity Faults (E0401–E0404)

Source: GSM Manager. Category D — no relay effect. Motor continues on local duty cycle.

| E-Code | String Name | Cat | LCD Row 1 | Phase LEDs | TRIP | Condition |
| --- | --- | --- | --- | --- | --- | --- |
| E0401 | GSM_CONN_LOST | D | GSM: Searching.. | Unchanged | 0 | GSM state machine drops connection. NET 1 LED off. Device runs on stored duty cycle. |
| E0402 | GSM_SIM_FAIL | D | SIM Error | Unchanged | 0 | AT+QCCID no response after 3 retries at boot. SIM not detected. |
| E0403 | GSM_NETWORK_FAIL | D | Network Fail | Unchanged | 0 | SIM present, no network registration. Coverage gap or RSSI too low. |
| E0404 | MQTT_CONN_LOST | D | MQTT Offline | Unchanged | 0 | GSM connected but MQTT broker unreachable. TLS cert expiry possible. |

6. AWS IoT Core Rules — Production

| Rule Name | SQL Filter | Actions | Lambda / Purpose |
| --- | --- | --- | --- |
| smarthika_telemetry_ingest | SELECT * FROM 'gw/+/telemetry' | Timestream + DynamoDB shadow | compute_ddl_and_drawdown. check_phase_leds. Publishes display on threshold breach. |
| smarthika_status_events | SELECT * FROM 'gw/+/status' | DynamoDB + SNS on motor change | capture_sdl on idle. update_mode_leds. Publishes display payload on mode/motor change. |
| smarthika_health_monitor | SELECT * FROM 'gw/+/health' | DynamoDB fleet table | check_ota_version. update_net_leds from boot fields. Initiates OTA if fw version mismatch. |
| smarthika_fault_alerts | SELECT * FROM 'gw/+/error' | SNS (SMS) + Lambda | handle_fault. Maps E-code to display TRIP+LCD. Sends push+SMS per role notification matrix. |
| smarthika_cmd_responses | SELECT * FROM 'gw/+/response' | DynamoDB command_log | process_cmd_response. Matches request_id. Display update on ok. NACK to display.lcd on rejected. |
| smarthika_ota_events | SELECT * FROM 'gw/+/ota' | DynamoDB + Lambda | handle_ota_state. Manages display LCD overrides across all OTA state transitions. |
| smarthika_recovery_log | SELECT * FROM 'gw/+/recovery' | Timestream + Lambda | compute_recharge_rate. Fits recovery curve. Publishes DUTY_CYCLE_UPDATE on 90min completion. |
| smarthika_yield_test_stream | SELECT * FROM 'gw/+/yieldtest' | Timestream + Lambda | analyze_yield_step. Updates display LCD live. Runs Jacob+Theis analysis on final step. |
| smarthika_network_test | SELECT * FROM 'gw/+/networktest' | Timestream + Lambda | compute_inter_bore_transmissivity. Updates aquifer_map DynamoDB table. |
| smarthika_env_nodes | SELECT * FROM 'nodes/+/env' | Timestream | Soil and weather storage for ET and irrigation scheduling calculations. |
| smarthika_valve_status | SELECT * FROM 'valves/+/status' | DynamoDB valve state | Valve state machine updates in DynamoDB. |

7. Schema Version Changelog

| Version | Date | Changes |
| --- | --- | --- |
| v1.0 | Original | Initial schema. telemetry, status, health, error, response, cmd. Basic electrical and motor fields. |
| v2.1 | Feb 2026 | Added: el.pf, el.tp, el.imb, m.srt, m.dry flags. Phase angle support. sys.bp. snap in error. All 3 phase currents. Discharge pressure hy.p. |
| v3.0 | Apr 2026 | NEW TOPICS: gw/{id}/display (Cloud→App), gw/{id}/ota, gw/{id}/yieldtest, gw/{id}/recovery, gw/{id}/networktest. HEALTH LEAN: removed sys.ac, conf.mode, net.t. Added boot.* fields. Moved sys.bv, sys.bp, net_s to telemetry 60s gate. ERROR: added e_code (E-format), cat field. CMD: added farm_id, user_id, request_id. STATUS: added active_test, trigger. RESPONSE: added reason_code. All payloads carry "v":"3.0". |

SMARTHIKA AGRO  •  MQTT Payload Reference  •  Schema v3.0  •  April 2026

Elcinotech Solutions Pvt. Ltd.  •  GRC_SMARTHIKA_SPI_VI_V3  •  Internal — Not for Distribution